EXERCISE 1: Two-line Elements (TLEs)

| Satellite Catalog ID | COSPAR International Designator | Satellite Name | Object Type | Operating Country | Epoch (Calendar Date) | Semi-Major Axis (a) [km] | Eccentricity (e) | Inclination (i) [deg] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22566 | 1993-016B | SL-16 R/B | ROCKET BODY | CIS | 2015-06-06 | 7220.778 | 0.0010719 | 071.0065 |
| 30239 | 1999-025WY | FENGYUN 1C DEB | DEBRIS | PRC | 2017-10-01 | 8498.051 | 0.1501976 | 99.7201 |
| 44625 | 2019-067B | MEV-1 | PAYLOAD | US | 2019-11-16 | 45721.973 | 0.4204334 | 005.8940 |
| 47944 | 2021-022N | ELSA-D | PAYLOAD | JPN | 2022-11-28 | 6915.464 | 0.0016455 | 97.5125 |
| 27378 | 2002-006A | ECHOSTAR 7 | PAYLOAD | US | 2023-06-11 | 42288.125 | 0.0006061 | 1.1620 |

For semi-major axis calculation, I used the formula for Kepler’s 3rd law and calculated in Desmos:

n=μa3

By solving for the semi-major axis a and converting from revolutions per day to radians per second, the semi-major axis can be found. μ is Earth’s gravitational parameter, 398600.4418 km cubed per second squared, and n is the mean motion located in the TLE.

For the epoch date in calendar form, I used https://ensatellite.com/tle-epoch-to-date/ to determine the day value for each TLE from the fraction.

Table from CSV output of code:

| Satellite catalog ID | COSPAR ID | Satellite name | Object type | Operating country | Epoch (calendar date) | Semi-major axis, a (km) | Eccentricity, e | Inclination, i (deg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22566 | 93016B | SL-16 R/B | ROCKET BODY | 🇷🇺 | 6/6/15 | 7220.77597 | 0.0010719 | 71.0065 |
| 30239 | 99025WY | FENGYUN 1C DEB | DEBRIS | 🇨🇳 | 10/1/17 | 8498.047936 | 0.1501976 | 99.7201 |
| 44625 | 19067B | MEV-1 | PAYLOAD | 🇺🇸 | 11/16/19 | 45721.95607 | 0.4204334 | 5.894 |
| 47944 | 21022N | ELSA-D | PAYLOAD | 🇯🇵 | 11/28/22 | 6915.461764 | 0.0016455 | 97.5125 |
| 27378 | 02006A | ECHOSTAR 7 | PAYLOAD | 🇺🇸 | 6/11/23 | 42288.10907 | 0.0006061 | 1.162 |

These data points match the manually determined ones based on the same TLEs! For the coding, I used ChatGPT to assist in writing and debugging my code along with using code from a previous TLE extractor tool I developed in a previous course.