EXERCISE 1: Two-line Elements (TLEs)

| Satellite Catalog ID | COSPAR International Designator | Satellite Name | Object Type | Operating Country | Epoch (Calendar Date) | Semi-Major Axis (a) [km] | Eccentricity (e) | Inclination (i) [deg] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22566 | 1993-016B | SL-16 R/B | ROCKET BODY | CIS | 2015-06-06 | 7220.776 | 0.0010719 | 071.0065 |
| 30239 | 1999-025WY | FENGYUN 1C DEB | DEBRIS | PRC | 2017-10-01 | 8498.048 | 0.1501976 | 99.7201 |
| 44625 | 2019-067B | MEV-1 | PAYLOAD | US | 2019-11-16 | 45721.956 | 0.4204334 | 005.8940 |
| 47944 | 2021-022N | ELSA-D | PAYLOAD | JPN | 2022-11-28 | 6915.462 | 0.0016455 | 97.5125 |
| 27378 | 2002-006A | ECHOSTAR 7 | PAYLOAD | US | 2023-06-11 | 42288.109 | 0.0006061 | 1.1620 |

For semi-major axis calculation, I used the formula for Kepler’s 3rd law and calculated it in Desmos:

n=μa3

By solving for the semi-major axis a and converting from revolutions per day to radians per second, the semi-major axis can be found. μ is Earth’s gravitational parameter, 398600 km cubed per second squared, and n is the mean motion from the TLE.

For the epoch date in calendar form, I used https://ensatellite.com/tle-epoch-to-date/ to determine the day value for each object from the epoch fraction in the TLE.

OPTION A - Table from CSV output of code:

| Satellite catalog ID | COSPAR ID | Satellite name | Object type | Operating country | Epoch (calendar date) | Semi-major axis, a (km) | Eccentricity, e | Inclination, i (deg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22566 | 93016B | SL-16 R/B | ROCKET BODY | 🇷🇺 | 6/6/15 | 7220.77597 | 0.0010719 | 71.0065 |
| 30239 | 99025WY | FENGYUN 1C DEB | DEBRIS | 🇨🇳 | 10/1/17 | 8498.047936 | 0.1501976 | 99.7201 |
| 44625 | 19067B | MEV-1 | PAYLOAD | 🇺🇸 | 11/16/19 | 45721.95607 | 0.4204334 | 5.894 |
| 47944 | 21022N | ELSA-D | PAYLOAD | 🇯🇵 | 11/28/22 | 6915.461764 | 0.0016455 | 97.5125 |
| 27378 | 02006A | ECHOSTAR 7 | PAYLOAD | 🇺🇸 | 6/11/23 | 42288.10907 | 0.0006061 | 1.162 |

These data points match the manually determined ones based on the same TLEs! For the coding, I used ChatGPT to assist in writing and debugging my code. My commented code can be found on Github under https://github.com/dgantt6/INTA4803-ClassAssignments/tree/master/Assignment1. It is also in the zip file in the submission.

The general process of my script follows the logic:

- Log-in to space-track.org with user credentials to access its satellite catalog

- Open the TLEs.txt file and extract each two lines

  - With each two lines, extract the elemental data pieces based on the indices of the element and the line they fall on

  - The NORAD ID and designator are taken directly from the TLE

  - For the epoch fraction, take the first two indices and if they’re less than 57, the date is 20XX. Otherwise, the date is 19XX, the rest of the epoch fraction is the time since January 1 of that year, which can be calculated using the datetime package. This is now the calendar date (not including seconds and milliseconds)

  - The inclination in degrees is taken directly from the TLE.

  - The eccentricity for elliptical orbits (assuming these satellites are in elliptical orbits) must be less than 1, and the number in the TLE is missing the leading zero which is added in the code.

  - For the semi-major axis (a), the mean motion given in the TLE in revolutions per day can be used to calculate a. The equations and mu value from part 1 of this exercise matches the equations and mu value used in the code.

  - All these values are inputted into a dictionary for further use after the loop

- Look up all the NORAD IDs in space-track.org

  - Based on common launching nations, use hard-coded list to map ISO-3 country codes to ISO-2 (for later use)

    - This list could be better refined so it is not hard-coded and can input any launching country rather than just popular countries.

  - Lookup all unique NORAD IDs from TLE file in space-track.org and store it (including extraction logic based on how the json data comes back)

  - Add the satellite name, object type, and country flag emoji (assuming it matches one of the common countries) to the dataset made in part 2

- Use pandas package to create an organized table of the data

- Export to CSV

EXERCISE 2: Protected Regions

SL-16 R/B:

- This rocket body flies in the orbital regime of LEO.

- This payload resides in a protected region of LEO according to the Inter-agency Debris Coordination Committee.

- While this rocket body doesn’t have station keeping capabilities, I would expect it to stay in its current orbit in the long term. As it flies between apogee of 848 km and perigee of 835 km currently, the rocket body will likely not degrade in its orbit as it is still very far from the altitude at which satellites burn up in Earth’s atmosphere.

FENGYUN 1C DEB

- This debris flies in the orbital regime of LEO and MEO.

- This debris intersects a protected region of LEO according to the Inter-agency Debris Coordination Committee.

- This debris intersects a protected region rather than resides in one.

MEV-1

- This payload flies in the orbital regime of GEO.

- This payload resides in a protected region of GEO according to the Inter-agency Debris Coordination Committee.

- This payload designed to extend the life of other GEO satellites will continue to stay in protected regions of GEO because it’s intended to extend the life of other satellites in the region.

ELSA-D

- This payload flies in the orbital regime of LEO.

- This payload resides in a protected region of LEO according to the Inter-agency Debris Coordination Committee.

- This payload designed to provide on-orbit servicing and debris removal capabilities completed its mission operations in 2024 and will therefore not stay in the protected region of LEO in the long term.

ECHOSTAR 7

- This payload flies in the orbital regime of GEO.

- This payload resides in a protected region of GEO according to the Inter-agency Debris Coordination Committee.

- This payload currently resides 122 km above its original GEO orbit and therefore remains in the protected region of GEO. However, it was supposed to end its mission 300 km above its original orbit and now has no fuel left to perform further station keeping getting it to a more suitable graveyard orbit. Due to the costliness of sending another satellite to push ECHOSTAR 7 to its intended graveyard orbit, it will likely remain in the protected region of GEO for the long term.

OPTION B:

The Federal Communications Commission (FCC) dates to 1934 when The Communications Act of 1934 was codified into law. This act created the FCC and gave the organization the authority to monitor and regulate the telephone, telegraph, and radio communication industries. Since its formation, the Act has been regularly updated based on new technologies such as satellite communications. Currently, CFR Part 25 highlights many regulations regarding the use of satellites in Earth orbit exhibiting radio communication behaviors. Specifically, 47 CFR 25.114(d)(14)(vii)(A) states that geostationary orbit satellites must disclose the altitude for their disposal orbit and the calculations for such orbit. This law applies to all geostationary spacecraft registered to the United States which must have been given FCC approval for radio communication privileges including disposal plans. Satellite disposal plans are included to preserve geostationary orbital slots, a limited natural resource, for future mission use. For geostationary orbits, the most accepted form of satellite disposal is pushing the satellite to an altitude more than 200 kilometers above the geostationary belt.

In 2002, DISH launched the geostationary EchoStar-7 with the FCC and later filed for an orbital debris mitigation plan in 2012. However, DISH’s fuel usage didn’t account for the necessary orbital transition and only ended up completing about 40% of its required altitude change, remaining well under its 300 kilometers promise. Because EchoStar-7 was registered under the US, the FCC retained the authority to fine DISH for its unsatisfactory performance in disposing adequately of EchoStar-7. This satellite, now defunct and unable to perform further maneuvering, is a cause of space debris and resides in a valuable slot in the geostationary orbital regime.

COSPAR ID pulled from space-track.org satellite catalog rather than directly from the TLE.

Inter-Agency Space Debris Coordination Committee Steering Group and Working Group 4, IADC Space Debris Mitigation Guidelines (2020), 7.

Space-Track.org, Two-Line Element–Derived Orbital Parameters for Earth-Orbiting Objects, United States Space Force, accessed January 21, 2026, https://www.space-track.org.

Northrop Grumman Achieves First-Ever Undocking Between Two Commercial Spacecraft in Geosynchronous Orbit, Northrop Grumman Press Release, April 9, 2025, Northrop Grumman, accessed January 21, 2026, https://news.northropgrumman.com/satellites/northrop-grumman-achieves-first-ever-undocking-between-two-commercial-spacecraft-in-geosynchronous-orbit.

Astroscale’s ELSA-d Finalizes De-Orbit Operations Marking Successful Mission Conclusion, Astroscale press release, January 24, 2024, Astroscale Holdings Inc., accessed January 21, 2026, https://www.astroscale.com/en/news/astroscales-elsa-d-finalizes-de-orbit-operations-marking-successful-mission-conclusion.

Federal Communications Commission, FCC Takes First Space Debris Enforcement Action: Settles Investigation of DISH for Failing to Comply with Deorbiting Plan, press release, October 2, 2023, accessed January 21, 2026, https://docs.fcc.gov/public/attachments/DOC-397412A1.pdf.

The Communications Act of 1934, Bureau of Justice Assistance, U.S. Department of Justice, accessed January 21, 2026, https://bja.ojp.gov/program/it/privacy-civil-liberties/authorities/statutes/1288.

47 C.F.R. § 25.114(d)(14)(vii)(A) (Code of Federal Regulations, “Applications for Space Station Authorizations”), accessed January 21, 2026, https://www.ecfr.gov/current/title-47/part-25#p-25.114(d)(14)(vii)(A).

Federal Communications Commission, FCC Takes First Space Debris Enforcement Action: Settles Investigation of DISH for Failing to Comply with Deorbiting Plan, press release, October 2, 2023, accessed January 21, 2026.